# PRELIMINARY TERRESTRIAL RISK ASSESSMENT

South Waste Rock Pile at the Sunro Mine Site, Jordan River, BC Teck Metals Ltd. June 5, 2024 AtkinsRéalis Ref: 669765

Signature Page

| Prepared By: |  |  |
| --- | --- | --- |
| Lindsay Du Gas, MET, RPBio Senior Risk Assessor / Toxicologist Environment Practice Engineering Services Canada |  | Lindsay Woof, MET, RPBio Risk Assessor/Toxicologist Environment Practice Engineering Services Canada |
| Reviewed By: |  |  |
| Tara Siemens Kennedy, MET, PChem, CSAP Senior Risk Assessor and Toxicologist Environment Practice Engineering Services Canada |  |  |

Notice

This report has been prepared and the work referred to in this report have been undertaken by AtkinsRéalis Canada Inc. (AtkinsRéalis) for the exclusive use of Teck Metals Ltd. (Teck), who has been party to the development of the scope of work and understands its limitations. The methodology, findings, conclusions and recommendations in this report are based solely upon the scope of work and subject to the time and budgetary considerations described in the proposal and/or contract pursuant to which this report was issued. Any use, reliance on, or decision made by a third party based on this report is the sole responsibility of such third party. AtkinsRéalis accepts no liability or responsibility for any damages that may be suffered or incurred by any third party as a result of the use of, reliance on, or any decision made based on this report. Should this report be submitted to the BC Ministry of Environment & Climate Change Strategy (ENV) by Teck, ENV is authorized to rely on the results in the report, subject to the limitations set out herein, for the sole purpose of determining whether Teck has fulfilled its obligations with respect to meeting the regulatory requirements of ENV.

The findings, conclusions and recommendations in this report (i) have been developed in a manner consistent with the level of skill normally exercised by professionals currently practicing under similar conditions in the area, and (ii) reflect AtkinsRéalis’ best judgment based on information available at the time of preparation of this report. No other warranties, either expressed or implied, are made as to the professional services provided under the terms of our original contract and included in this report. The findings and conclusions contained in this report are valid only as of the date of this report and may be based, in part, upon information provided by others. If any of the information is inaccurate, new information is discovered, site conditions change or standards are amended, modifications to this report may be necessary. The results of this assessment should in no way be construed as a warranty that the subject site is free from any and all environmental impact.

Any soil and rock descriptions in this report and associated logs have been made with the intent of providing general information on the subsurface conditions of the site. This information should not be used as geotechnical data for any purpose unless specifically addressed in the text of this report. Groundwater conditions described in this report refer only to those observed at the location and time of observation noted in the report.

This report must be read as a whole, as sections taken out of context may be misleading. If discrepancies occur between the preliminary (draft) and final version of this report, it is the final version that takes precedence. Nothing in this report is intended to constitute or provide a legal opinion.

The contents of this report are confidential and proprietary. Other than by Teck, copying or distribution of this report or use of or reliance on the information contained herein, in whole or in part, is not permitted without the express written permission of Teck and AtkinsRéalis.

Contents

Executive Summary	6

Human Health	6

Terrestrial Ecological Risks	6

1.	Introduction	8

1.1	Regulatory Framework	9

1.2	Previous Investigations and Risk Assessment	9

1.3	Approach and Guidance	11

2.	Problem Formulation	12

2.1	Site Setting	12

2.2	Ecological Setting	12

2.2.1	Surface Water Bodies	12

2.2.2	Regional Ecology	12

2.2.3	Species at Risk	14

2.3	Contaminants of Potential Concern	14

2.3.1	Soil COPC Screening	14

2.4	Receptors of Concern and Exposure Pathways	16

2.4.1	Human Receptors of Concern and Exposure Pathway Analysis	16

2.4.2	Ecological Receptors of Concern and Exposure Pathway Analysis	17

2.5	Conceptual Site Model	18

3.	Preliminary Terrestrial Risk Assessment	19

3.1	South Waste Rock Pile	19

4.	Conclusions	22

4.1	Human Health Risks	22

4.2	Terrestrial Ecological Risks	22

5.	References	24

In-Text Tables

Table 1-1:	Summary of Areas of Environmental Concern	10

Table 2-1:	Summary of Soil COPCs	15

Table 2-2:	Conceptual Site Model for Human Health	18

Table 2-3:	Conceptual Site Model for Ecological Receptors	18

Contents (Cont’d)

Tables

- Soil Log

- Summary of Analytical Results for Metals in Soil

- Soil Sample Location Plan

Drawings

- 631848-101 – Site Location Plan

- 631848-102 – Areas of Environmental Concern

- 631848-103 – Soil Sample Location Plan

Appendices

- Habitat Assessment and Species Search Output

- ProUCL Output

# Executive Summary

AtkinsRéalis Canada Inc. (AtkinsRéalis) has completed this Preliminary Terrestrial Risk Assessment (PTRA) for Teck Metals Ltd. (Teck), for the South Waste Rock Pile at the former Sunro copper mine (the “Site”) near Jordan River, BC. The purpose of the PTRA was to evaluate the potential for exposures and risks to people using this Site, as well as to terrestrial ecological receptors, from metals identified in soil. The results of the PTRA will be used to support risk management and remediation decision-making for this area of the Site. The scope of the PTRA is limited to the South Waste Rock Pile. The PTRA does not include an assessment of the North Waste Rock Pile, Concentrate Storage Area and tailings at the Site (including an area of piled tailings in the South Waste Rock Pile AEC) as, based on the available data and experience at similar sites, these areas are likely to pose unacceptable risks for human and/or ecological receptors and are also a source of metal loading to the adjacent aquatic environment of the Jordan River.

## Human Health

The Site is accessible via an unpaved forest access road. While there is no formal day use area, signs of recreational use have been noted during Site visits. It was assumed that members of local Indigenous communities, as well as members of the general public, could use the Site for traditional or recreational purposes. No contaminants of potential concern (COPCs) for human health were identified in surface soil at the South Waste Rock Pile area of environmental concern (AEC), therefore no unacceptable risks to human health resulting from direct exposures to soil in this area are anticipated. Further, given the lack of COPCs in surface soil, the potential for significant exposures to soil COPCs via indirect exposure, including via consumption of country or traditional foods, is considered to be negligible.

## Terrestrial Ecological Risks

The Site provides significant terrestrial habitat for a diverse array of species, potentially including species at risk (SAR). COPCs for terrestrial ecological receptors were identified in surface soil (copper and selenium) and subsurface soil (copper) at the South Waste Rock Pile, with copper identified as the COPC likely to drive risks.

An assessment of the terrestrial habitat at the South Waste Rock Pile was completed by Corvidae (2021). While elevated concentrations of copper have been identified in soil in this area, observations from SNC-Lavalin (2016) and Corvidae (2021) suggest that mature riparian forest ecosystem is present and was described as “intact” (Corvidae, 2021). While not conclusive, the biological observations for the South Waste Rock Pile AEC suggest that effects to terrestrial ecological receptors are not likely to be occurring at the population or community level. Further assessment of the area relative to reference areas could be conducted to confirm this finding.

The habitat assessment also identified potentially suitable habitat in the riparian corridor that bisects the North and South Waste Rock Piles for the Northern red-legged frog, an amphibian SAR documented within 1.4 km of the Site. Additionally, this habitat may also be suitable for the Wandering salamander, another amphibian SAR with documented occurrences in the area surrounding the Site. The presence of these amphibian SAR at the South Waste Rock Pile AEC is unknown and requires confirmation. The areas of suitable habitat for these amphibians in the AEC described by Corvidae (2021) relative to the areas being remediated within the AEC is also unclear. As a result, it is recommended that a survey of the riparian corridor be conducted to determine if the amphibian SAR are present, and that detailed mapping of habitat considered to be potentially suitable for SAR amphibians be performed. If these species are directly observed or are determined to have a high likelihood of being present based on the suitability of habitat within the contaminated area, additional assessment, followed by the completion of a Detailed Ecological Risk Assessment, is recommended.

Given the relatively small size of the areas of the Site evaluated herein and the vast productive habitat that surrounds these areas, consideration should be given to the disturbance to ecological receptors that would be required to complete physical remediation at the relatively inaccessible areas of the Site. Additionally, while the current PTRA does not include an evaluation of the potential for contributions of COPCs from these areas to nearby aquatic receiving environments, including the Jordan River, COPC loading from the South Waste Rock Pile should be considered in the decision-making process for contaminated soil management.

# Introduction

AtkinsRéalis Canada Inc. (AtkinsRéalis) has completed this Preliminary Terrestrial Risk Assessment (PTRA) for Teck Metals Ltd. (Teck), for the South Waste Rock Pile at former Sunro copper mine (the “Site”) near Jordan River, BC. The purpose of the PTRA was to evaluate the potential for exposures and risks to people using this Site, as well as to terrestrial ecological receptors, from metals identified in soil. The results of the PTRA will be used to support risk management and remediation decision-making for this area of the Site. This work was completed under purchase order 10560 and terms and conditions of the MSA TR00013 between Teck and AtkinsRéalis, as outlined in the work plan and cost estimate submitted to the Teck titled “2023 to 2024 Water Sampling and Risk Opinion Cost Estimate at the Sunro Mine, Jordan River, BC” (SNC-Lavalin, 2023).

The Site is located 3.5 km upstream of the community of Jordan River on Vancouver Island, BC and adjacent to the Jordan River (Drawing 631848-101). Where the Site borders the Jordan River, steep slopes and cliffs are present. Historical mining operations associated with the Site were accessed via a portal on the main haul tunnel (5100’ level) 1.2 km upstream of the community of Jordan River. Mine deposits are present on the east bank of the Jordan River, which are identified as areas of environmental concern (AECs) 1 and 2 and are referred to as the ‘North Waste Rock Pile’ and ‘South Waste Rock Pile’, respectively. Additional AECs include the Concentrate Storage Area (3), Tailings Pipeline (4), 5100 Portal (5), and 1963 Mine Blowout (6) (SNC-Lavalin, 2018). Drawings 631848-101 and 102, details the AECs and their location at the Site.

Mine wastes and soil have been characterized at these AECs during previous investigations (see Section 1.2), with elevated concentrations of select metals above applicable BC Contaminated Sites Regulation0F (CSR) Standards (i.e., contamination). Previously, a Preliminary Risk Opinion (PRO; SNC-Lavalin, 2016) consisting of an exposure pathway analysis was provided for the North Waste Rock Pile, South Waste Rock Pile, and Concentrate Storage AECs; however, while potentially operable exposure pathways were identified, the PRO did not quantify exposures and associated risks. The PRO had also provided a qualitative assessment of potential risks to aquatic receptors, with a subsequent Preliminary Aquatic Ecological Risk Assessment completed by Azimuth Consulting Group Partnership (Azimuth, 2019).

The focus of the current PTRA is to further assess potential exposures and risks to human and ecological receptors associated with the soil contaminants identified at the South Waste Rock Pile. The scope of the PTRA is limited to the South Waste Rock AEC. The PTRA does not include an assessment of the North Waste Rock Pile, Concentrate Storage Area and tailings at the Site (including an area of piled tailings in the South Waste Rock Pile AEC) as, based on the available data and experience at similar sites, these areas are likely to pose unacceptable risks for human and/or ecological receptors and are also a source of metal loading to the adjacent aquatic environment of the Jordan River. As a result, these areas will have their own risk management plans/remediation plans.

Exposures to metals at the Site via the direct soil exposure pathways (i.e., incidental soil ingestion, skin contact and inhalation of particulates) are likely to drive risks to both human and ecological receptors and were the focus of this PTRA. A relatively small contribution from exposure to metals in other media (e.g., groundwater, sediment, surface water) would be anticipated for terrestrial receptors (including humans and terrestrial ecological receptors); therefore, additional routes of exposure were not evaluated herein. The potential for indirect exposure to mine-related contaminants though harvesting and ingestion of country foods (e.g., edible vegetation and/or wildlife) will be qualitatively discussed in the current evaluation.

The PTRA includes a problem formulation, comprised of the identification of contaminants of potential concern (COPCs) (i.e., based on receptor-pathway specific standards), receptors of concern, and the pathways by which receptors have the potential to be exposed to the COPCs. The PTRA also includes the quantitative evaluation of risks to humans (if applicable), using the Site for recreational or traditional purposes. Further, it evaluates the potential for unacceptable risks to terrestrial ecological receptors at the community or population level and considers risks to species at risk (SAR).

A limited biological survey/habitat assessment was conducted in July 2021 by Corvidae Environmental Consulting Inc. (Corvidae) to inventory and assess wildlife and vegetation in the waste rock area, including at the South Waste Rock Pile (Corvidae, 2021; Appendix A). The results of this assessment were reviewed and considered in the interpretation of the potential for soil quality at the Site to presents a potential risk to terrestrial ecological receptors at the Site. Additionally, Corvidae (2021) reviewed the potential for SAR to be present at the Site; while no SAR were identified during the Site visit, suitable habitat for listed amphibians was identified in the riparian corridor that bisects the South Waste Rock Pile. The implications of this will be considered in the determination of whether the evaluation of risks at the community to population level, versus the individual level, is appropriate for amphibians.

## Regulatory Framework

The substances at the Site exceeding the applicable standards/guidelines (i.e., contaminants of concern [COCs]) were identified by SNC-Lavalin (2018). All COCs previously identified at the Site were carried forward in the PTRA as preliminary COPCs. The COC in soil at the Site were assessed by analysis of soil and comparison of results to the reverted Wildlands (WLR) standards outlined in the following:

Contaminated Sites Regulation (CSR), B.C. Reg. 375/96, includes amendments up to B.C. Reg. 133/2022, March 1, 2023.

## Previous Investigations and Risk Assessment

AtkinsRéalis completed investigations from 2015 to 2018 to enhance the understanding of conditions at the Site in support of risk management and remediation planning, completing three iterations of site investigation reports, as follows:

Supplemental Stage 1 and Limited Stage 2 Preliminary Site Investigation (PSI) and Preliminary Risk Opinion, dated June 24, 2016 (SNC-Lavalin, 2016; the “2016 Stage 2 PSI”);

Limited Detailed Site Investigation (DSI), Sunro Mine, Jordan River, BC dated July 10, 2017 (SNC-Lavalin, 2017; the “2017 DSI”);

Supplemental Detailed Site Investigation, Sunro Mine, Jordan River, BC dated July 27, 2018 (SNC-Lavalin, 2018; the “2018 SDSI”); and

Preliminary Aquatic Ecological Risk Assessment – Jordan River adjacent to the 5100 Portal Area of the former Sunro Mine, BC dated August 2019 (Azimuth Consulting Group Partnership [Azimuth], 2019).

SNC-Lavalin identified on-Site and off-Site areas of potential environmental concern (APECs) with mine waste associated with the former operation for the Sunro mine as part the 2016 Stage 2 PSI. The Stage 2 PSI comprised soil, sediment, porewater and surface water sampling to identify AECs at the Site. Based on the results of the Stage 2 PSI, a PRO was conducted to qualitatively assess the potential for operable exposure pathways and thus potential risks to human, terrestrial and aquatic receptors. The PSI and PRO were intended to support regulator engagement for the Site.

During a desktop review, 13 suspected mine features were identified at the Site and seven were identified off-Site. Following the Site visit, six on-Site and six off-Site mine features were retained as APECs. Based on limited sampling results, six APECs were designated AECs, as described in Table 1-1 below. The AECs at this Site are shown on Drawing 631848-102.

Table 1-1:	Summary of Areas of Environmental Concern

| AEC | Description |
| --- | --- |
| 1 | North Waste Rock Pile: located adjacent to the Jordan River adjacent to the 5100 Portal. |
| 2 | South Waste Rock Pile: located adjacent to the Jordan River downstream from the 5100 Portal. |
| 3 | Concentrate Storage: located adjacent to Unnamed Creek #2 and the mine access road to the 5100 Portal. |
| 4 | Tailings Pipeline: plastic pipe containing tailings assumed to extend from the 5100 Portal along the mine access road to the Site boundary where it becomes an off-Site APEC. |
| 5 | 5100 Portal: Closed portal discharging water from the underground workings into the Jordan River. |
| 6 | 1963 Mine blowout to surface extending to the Jordan River past AEC 3 and north of the 5100 Portal Area. |

The investigation reports summarize the elevated concentrations of metals in mine wastes and soils, acid generation and leaching of metals from waste rock and/or tailings, as well as the discharge of impacted groundwater and surface water into aquatic receiving environments, including the Jordan River. Azimuth (2019) included a preliminary evaluation of risks to the Jordan River.

The investigations have identified mine wastes containing concentrations of metals exceeding the BC CSR Standards and Protocol 11 Upper Cap Concentrations (UCC) (BC Ministry of Environment and Climate Change Strategy (ENV), 2023a) for the protection of human health and/or environmental health in soil, groundwater, porewater and surface water at the Site. This included potential exposures to UCCs in soil mine wastes, porewater and surface water at the North Waste Rock Pile (AEC 1); soil at the South Waste Rock Pile (AEC 2); and soil only the Concentrate Storage Area (AEC 3) of the Site. Protocol 12 “high risk” conditions (i.e., where UCCs exceedances and complete exposure pathways per BC ENV [2023b] Protocol 12 are present) were identified at the Site in soil/mine wastes for the protection of ecological receptors only. On this basis, the Site is classified as a “high-risk” Site based on the current evaluation of the exposure pathways. At the North Waste Rock Pile, for example, the 95% upper confidence limit of the mean copper concentration was more than five times greater than the UCC (SNC-Lavalin, 2018). As assessment of the loading of dissolved copper from the Site to Jordan River was also completed by SNC-Lavalin (2018) to estimate the mass of copper discharging into the river. Based on the “high risk” conditions at the Site, proximity to the Jordan River and subsequent copper discharge, as well as presence of high sulphide waste rock, low sulphide waste rock and tailings at the North Waste Rock Pile and Concentrate Storage Area, these areas of the Site are to be remediated in the future. The tailings present at South Waste Rock Pile are also slated for remediation.

A biological survey/habitat assessment was also conducted in July 2021 by Corvidae Environmental Consulting Inc. (Corvidae) to inventory and assess wildlife and vegetation in the waste rock area, including in the South Waste Rock Pile (Corvidae, 2021).  The assessment reviewed the potential for SAR to be present at the Site. While no SAR were identified, Corvidae (2021) concluded that there is suitable habitat for listed amphibians in the riparian corridor that bisects the North and South Waste Rock Piles.

## Approach and Guidance

The PTRA has been completed in accordance with guidance and principles for risk assessment prescribed by the BC ENV CSR and associated guidance documents and protocols, as well as by the Canadian Council of Ministers of the Environment (CCME) and Health Canada. Risk assessment methods described in the guidance used are consistent with methods commonly used by regulatory agencies across Canada and the United States, including the United States Environmental Protection Agency (US EPA). The PTRA includes consideration of potential health risks to persons spending time at the Site, either directly or indirectly exposed to impacts reported in soil (i.e., the only media under evaluation in the current PTRA) as per BC ENV (2023c) and Health Canada (2010 and 2021). Ecological receptor exposure via soil pathways was evaluated accordance with BC ENV (2023c) and CCME (2020).

# Problem Formulation

The Problem Formulation identifies the Site setting, COPCs, receptors of concern (ROCs), and the potentially operable exposure pathways between the COPCs and the receptors.

## Site Setting

The historical mine comprising the Site, located on the southwest coast of Vancouver Island adjacent to the Jordan River and 3.5 km north of the community of Jordan River, encompasses an area of 250 hectares.

As previously described, historical mine operations occurred throughout the Site. As the Site is expected to naturalize overtime, the current and anticipated future land use of all AECs within the Site, including the South Waste Rock Pile, is reverted wildlands, associated with the application of BC CSR WLR standards. The Site is accessed via an unpaved forest access road, which merges with Highway 14. There is no day use area, however, indications of recreational use, as evident by the presence of makeshift seating, beer bottles and ammunition casings, have been noted during Site visits. Fishing may also occur in the Jordan River and recreational foraging for vegetation/fungi may also take place in the region.

## Ecological Setting

### Surface Water Bodies

The nearest surface water body is the Jordan River, which is adjacent to the Site and flows into the Pacific Ocean (Strait of Juan de Fuca). Additionally, two unnamed creeks have been observed flowing over waster rock and tailings on either side of the South Waste Rock Pile, which subsequently flow into the Jordan River. Theses creeks flow year-round and are referred to as unnamed Creek #1 and Creek #2. The current PTRA includes the evaluation of the potential for risks to human and ecological receptors associated with direct or indirect terrestrial exposures to COPCs at the South Waste Rock Pile only. While the current PTRA does not include an evaluation of the potential for contributions of COPCs from these areas to nearby aquatic receiving environments, including the Jordan River, COPC loading from these areas should be considered in the decision-making process for how to manage or remediate soils at the South Waste Rock Pile.

### Regional Ecology

The Site is located within the Coastal Western Hemlock very wet maritime (CWHvm1) subzone of BC. As stated in Meidinger and Pojar (1991), the CWHvm1 has a wet, humid climate with cool summers and mild winters with relatively little snow. Precipitation is high. Forests are typically dominated by Western hemlock and Western red cedar, as well as Amabilis fir and Yellow-cedar in wetter parts. The CWHvm1 understory generally features a well-developed shrub layer dominated by red huckleberry and Alaskan blueberry, and a well-developed moss layer dominated by Hylocomium splendens and Rhytidiadelphus loreus. Herbs are sparse and include minor amounts of Deer fern, Five-leaved bramble, Bunchberry, and Queen's cup. Subdued terrain on the west coast and northern end of Vancouver Island features very old successional stages of forest.

Wildlife that may be present include black-tailed deer, black bear, Douglas squirrel, Deer mouse and Brown bat. An incredible diversity of bird species is present, such as Blue grouse, Lewis woodpecker, American robin, Purple finch, Spotted owl, Northern flicker, Common raven, Chestnut-backed chickadee and Winter wren. Amphibian species may also be present in the damp litter of mature forests and near surface water, including Northwestern salamander, Western toad and Northern red-legged frog.

#### Habitat Assessments

The habitat assessments described below were conducted for the larger Site. Although the focus of the PTRA is the South Waste Rock Pile, observations provided for areas outside of the South Waste Rock Area are provided for context and information on surrounding available habitat that may influence the presence of wildlife species at the South Waste Rock Pile.

A Site visit was completed by SNC-Lavalin in November 2015 by a Registered Professional Biologist. Generally, the natural vegetation at the Site was noted to have been significantly modified by historical development and excavation activities. The dominant climax species in areas outside of the waste rock piles, including the South Waste Rock Pile, consisted primarily of Western hemlock followed by lesser amounts of Western red cedar. The South Waste Rock Pile had an abundance of tree cover, shrub layer, and undergrowth. Established stands included Western hemlock, Western red cedar, and the occasional Big-leaf maple, the shrub layer consisted of Salal, Oval-leafed blueberry and Swordfern thickets.

The ecological setting, as well as the identification of the potential use of the Site by species at risk, was described and evaluated by Corvidae (2021). Their findings are summarized in a letter report with photos, provided as Appendix A. A Site visit was completed in July 2021 to document the ecological features of the waste rock areas, such as vegetation and habitat types and wildlife presence. The areas of waste rock described in the report, which appears to include areas within both the North and South Waste Rock Pile AECs (as it was not specified in the report), was found to be mostly barren, consisting of cemented rock with no vegetation growth and erosion of neighbouring soils into the Jordan River. A riparian corridor is present between the waste rock and an unnamed tributary to the Jordan River, which was occupied by Western hemlock and Western redcedar, Salmonberry, Sword fern and Huckleberry, while wetland vegetation was present in the stream and on the stream boundary. The extent and the size of this corridor and the overlap with identification contamination at the South Waste Rock Pile AEC have not yet been documented and should be determined through future habitat assessment of this area. Numerous wildlife species are noted to inhabit the area, however, the piled and compacted waste rock is not suitable for habitat due to the steep slope and barren nature. The forest riparian areas however are suitable for wildlife species, such as birds, mammals, reptiles and amphibians. Bear and deer scat were observed on Site, as well as American robin, Chipping sparrow, Dark-eyed junco and Red-tailed hawk. Critical habitat for Little Brown myotis (Myotis lucifugus) and Northern myotis (Myotis septentrionalis) were noted to overlap with the Site, and roosting habitat in the forest (on cedar snags) was also observed.

### Species at Risk

No species at risk were observed during the 2021 wildlife and vegetation survey (Corvidae, 2021). A query of the BC CDC iMap tool by Corvidae (2021) yielded occurrences of Northern red-legged frog (Rana aurora) approximately 1.4 km to the east of the waste rock area in the Desolation Creek watershed and in the ditch near the forest service road. Corvidae (2021) also concluded that the riparian corridor that bisects the waste rock area in the South Waste Rock Pile is considered suitable habitat for the Northern red-legged frog. A BC CDC iMAP (2023) query was also completed herein, which identified the same species (occurrence 32313) as well as potential presence of ermine (Anguinae sub species; occurrence 126698) and Wandering salamander (Neides vagrans; occurrence 103421) in the Jordan River area. See Appendix A for the 2023 species search output.

## Contaminants of Potential Concern

SNC-Lavalin (2016, 2018) provides a summary of previous investigations at the Site, which includes the collection and analysis of soil samples from the Site in 2016 to 2018. Additionally, groundwater, sediment, porewater and surface water collected from the Site were also characterized. As previously described and further documented in Section 2.4, the scope of the current PTRA is limited to the evaluation of potential risks to human and terrestrial ecological receptors associated with exposure to contamination in soils at the South Waste Rock Pile. The area of piled tailings within the South Waste Rock Pile AEC (see Drawings 631848-102 and 103) has been excluded from evaluation, as this area will be physically remediated. While the current PTRA does not include an evaluation of the potential for contributions of COPCs from these areas to nearby aquatic receiving environments, including the Jordan River, COPC loading from these areas should be considered in the decision-making process for how to manage or remediate soils at the South Waste Rock Pile at the Site.

### Soil COPC Screening

All metals identified at concentrations that exceeded the BC CSR WLR numerical standards in soil at the Site were retained as preliminary COPCs for pathway specific screening in the PTRA to identify final COPCs. Soil concentrations of preliminary COPCs were compared to BC ENV CSR Schedule 3.1 Standards protective of human health for human intake of contaminated soil and for the environment for toxicity to soil invertebrates and plants. As noted, the most relevant land use to be evaluated at the Site, as defined in the CSR, and considered for this PTRA included WLR. Soil parameters in exceedance of the CSR standards for the protection of human health and terrestrial ecological receptors, as well as the BC ENV Protocol 4 (BC ENV, 2023d) background concentrations, were retained as final COPCs requiring further assessment and were carried forward into the PTRA (Section 3), when operable exposure pathways to receptors were identified. CSR Industrial Land Use (IL) standards apply to depths of 3 m and greater and thus were used to screen for COPCs in soil samples collected beyond this depth.

All available soil sample locations located within the South Waste Rock Pile (AEC 2) were included the soil dataset for the current assessment which included 20 soil samples, 13 of which were for surface soil (≤ 1 m bgs).

Final soil COPCs are summarized below in Table 2-1 for the South Waste Rock Pile (AEC2).As presented in Table 2-1, no final COPCs were identified for the evaluation of risks to human health. Final COPCs identified for the evaluation of risks to terrestrial ecological ROCs include copper and selenium.

Table 2-1:	Summary of Soil COPCs

| Preliminary COPC | Maximum Soil Concentration (µg/g) (and Location) | Protocol 4 Background Concentration (µg/g) | CSR UCC |  | CSR Screening Standards |  | Retained as a Final COPC for: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Human Health (µg/g) | Terrestrial Ecological (µg/g) | Human Health (µg/g)a | Terrestrial Ecological (µg/g)a | Human Health? | Terrestrial Ecological? |
| AEC 2 South Waste Rock Pile, Surface Soil (≤ 1 m bgs) |  |  |  |  |  |  |  |  |
| Chromium | 43 (BH18-10) | 65 | 2,500 | 2,000 | 250 | 200 | No | No |
| Cobalt | 21.7 (WR15-05) | 30 | 250 | 450 | 25 | 45 | No | No |
| Copper | 4,020 (SS17-07) | 100 | 75,000 | 1,500 | 7,500 | 150 | No | Yes |
| Iron | 46,300 (SS17-08) | 70,000 | 350,000 | NS | 35,000b | NS | No* | No* |
| Selenium | 5.73 (SS17-07) | 4 | 4,000 | 15 | 400 | 1.5 | No | Yes |
| Vanadium | 74.9 (BH18-03) | 200 | 4,000 | 1,500 | 400 | 150 | No | No |
| AEC 2 South Waste Rock Pile, Subsurface Soil (> 1 m bgs) |  |  |  |  |  |  |  |  |
| Chromium | 103 (BH18-03 DUP) | 65 | 2,500 | 2,000 | 250 | 200 | No | No |
| Cobalt | 28.6 (BH18-02) | 30 | 250 | 450 | 25 | 45 | No* | No |
| Copper | 1,510 (BH18-02) | 100 | 75,000 | 1,500 | 7,500 | 150 (300c) | No | Yes |
| Iron | 62,500 (BH18-03) | 70,000 | 350,000 | NS | 35,000b | NS | No* | No* |
| Selenium | 0.75 (BH18-10) | 4 | 4,000 | 15 | 400 | 1.5 | No | No |
| Vanadium | 237 (BH18-03 DUP) | 200 | 4,000 | 1,500 | 400 | 150 (300c) | No | No |

Notes:

Bold	Indicates parameter retained as a COPC for further evaluation in the HHERA.

Underlined	Exceeds CSR UCC

NS	No standard

UCC	Upper Cap Concentration, BC CSR Protocol 11

WLR	Wildlands reverted land use

( )	Brackets, comparison to IL standard as sample is > 3 m bgs and exceeds WLR standard; if concentrations less than IL, not retained as a COPC

a	CSR Schedule 3.1 Part 1 Matrix Standard, Soil Standard for human intake of contaminated soil or toxicity to soil invertebrates and plants, for WLR

b	CSR Schedule 3.2 Part 2 Generic Numerical Soil Standards to Protect Human Health

c 	CSR Schedule 3.1 Part 1 Matrix Standard, Soil Standard for human intake of contaminated soil or toxicity to soil invertebrates and plants, for IL > 3 m bgs

* 	Maximum concentration below Protocol 4 background concentration in soil for Vancouver Island

## Receptors of Concern and Exposure Pathways

### Human Receptors of Concern and Exposure Pathway Analysis

Potential receptors of concern (ROCs) considered for evaluation in the current PTRA were based on assumed current/future uses at the Site and are consistent with ENV and Health Canada (2021) guidance. As previously described, the Site is accessed via an unpaved forest access road, which merges with Highway 14. Access is not restricted, and while there is no formal day use area, signs of recreational use including makeshift seating, beer bottles and ammunition casings, have been noted during Site visits. Potential receptor groups at the Site associated with wildlands use were considered for further evaluation in the PTRA, including:

Members of local Indigenous communities; and

Recreational land users.

From the list of human ROCs above, the groups with the highest potential exposure to COPCs at the Site were selected for evaluation of potential risk where operable exposure pathways were identified. Those selected receptors serve as surrogates for the groups that are not directly assessed. Justification for retaining and excluding receptor groups is presented for each group, below:

Members of Local Indigenous Communities: Retained.

The Site is located within the ancestral land of the Pacheedaht First Nation, therefore members of local Indigenous communities were retained as ROCs. People may be exposed to COPCs present at depths ≤ 1 m bgs through direct contact with soil including incidental ingestion, dermal contact and soil particulate (i.e., dust) inhalation. Ultimately, no soil COPCs were identified for the protection of human health therefore no unacceptable risks to this receptor group are predicted.

Recreational Land User: Exposures assumed to be similar or less than those for members of local Indigenous Communities.

At this time, it was assumed that Indigenous community member use of the Site would be similar to or greater than that of a recreational land user. As such,  as with the local Indigenous communities, no unacceptable risks are predicted for this receptor group.

It is acknowledged that human receptors at the Site could be exposed to COPCs in other media or via other pathways that are not directly or evaluated in the current PTRA; however significant exposures are not anticipated. This includes consumption of surface water, as well as consumption of traditional or country foods harvested from the Site, such as during hunting, fishing and foraging. As no COPCs were identified in soil for the protection of human health, significant plant uptake of contaminants from soil is considered unlikely. As such, bioaccumulation of contaminants up the food chain with subsequent human exposure is considered to be an insignificant exposure pathway. Based on the lack of identification of soil COPCs for the evaluation of risks to human health at the South Waste Rock Pile, the potential for human health risks is considered to be negligible based on the currently available data. This conclusion is considered appropriate to support risk management and remediation decision-making for the South Waste Rock Pile AEC.

### Ecological Receptors of Concern and Exposure Pathway Analysis

As per ENV Protocol 1 (2023c) and CCME (2020), ERAs are conducted to assess the potential for contamination to pose an unacceptable level of risk to ecological receptors, with the goal for the continued presence of a biologically diverse, functional, self-sustaining and interdependent community or ecosystem. Where the potential for unacceptable risks is identified, remediation or risk management may be required to mitigate current or future risks.

Protection of ecological receptors at the community or population level is important for the continued existence of individual species and the health of ecosystems as a whole. Though the primary focus of the ERA is to assess potential effects at the community or population level, in some circumstances, such as for species of special concern, protection at the individual level may be required.

For the ecological portion of the PTRA, listed and sensitive species, populations, communities, habitats or ecosystems that are potentially exposed to COPCs are defined as ROCs. Ecological ROCs have been selected based on the information compiled from field observations and desktop data reviews, as outlined in Corvidae (2021) and summarized in Section 2.2.

The following section identifies potential ecological ROCs and evaluates potentially operable exposure pathways at the Site, where soil COPCs were identified for the protection of terrestrial ecological receptors. Exposure pathways are the route(s) by which ecological receptors are exposed to COPCs identified at the AECs.

#### Terrestrial Ecological Receptors and Exposure Pathways

As described in Section 2.2, the biogeoclimatic zone which spans the Site support a tremendous diversity and density of plants and wildlife. As per BC ENV (2023c) and CCME (2020), the following ecological receptor groups were considered for inclusion as ROCs in the terrestrial environment and thus, have been identified as preliminary terrestrial ROCs for evaluation in the PTRA:

Terrestrial vegetation communities (including trees, shrubs and herbs);

Soil invertebrate communities;

Terrestrial bird populations (including herbivores, insectivores and carnivores);

Terrestrial mammal populations (including herbivores, insectivores and carnivores);

Reptiles;

Amphibians; and

Federal/provincially-listed species at-risk as individuals.

Three species of special concern were identified to either have been present in the vicinity of the Site, or to have the potential to be present at the Site. These species include the following:

Northern Red-legged Frog (Rana aurora, BC blue listed);

Ermine (Mustela richardsonii anguinae, BC blue listed); and

Wandering salmander (Aneides vagrans, BC blue listed).

Exposure pathways are the route(s) by which ecological receptors are exposed to contaminants in the environment (e.g., soil, water, food). The terrestrial ecological ROCs listed above have the potential to be directly exposed to COPCs identified in Site soils; the direct contact with soil COPC exposure pathway is anticipated to be the pathway that would contribute most significantly to a terrestrial organism’s overall exposure. Soil COPCs for terrestrial ecological receptors were identified at the South Waste Rock Pile AEC. Most ecological receptors are only anticipated to come into contact with COPCs at-grade (i.e., surface soil < 1 m bgs), with the exception of deep-rooting vegetation which may contact subsurface soil COPC up to a depth of < 3 m bgs (CSAP, 2013).

CCME (2020) provides potential ecological exposure pathways for the above-listed ROCs. Exposure pathways can be “direct” or “indirect”. Direct contact occurs when a receptor is exposed directly to the media containing the COPCs. An example would be a soil invertebrate (e.g., earthworm) exposed to COPCs in soil. An example of an indirect exposure would be an animal consuming prey that has accumulated COPCs from contaminated media. In the current PTRA, potential exposures and risks to terrestrial ecological receptors are qualitatively evaluated in Section 3.

## Conceptual Site Model

The conceptual models for the human health risk and ecological risk components of the PTRA are summarized in Table 2-2 and Table 2-3, respectively. Based on the information presented in the Problem Formulation, no COPCs were identified for the evaluation of risks to human receptors and thus, no unacceptable risks are predicted.

Based on the identification of final COPCs and potentially operable exposure pathways for terrestrial ecological receptors at the South Waste Rock Pile AEC, the PTRA includes a qualitative evaluation of risks to these receptors.

Table 2-2:	Conceptual Site Model for Human Health

| Land Use | Receptor of Concern | Age Group | Preliminary Risk Conclusion |
| --- | --- | --- | --- |
| Wildlands | Indigenous Community Members | All ages | No final COPCs identified and thus, no unacceptable risks are predicted |
|  | Recreational Land User | All ages | Exposure pathways and assumptions assumed to be equivalent to or less than those associated with Indigenous community members and thus, no unacceptable risks are predicted |

Note:

BOLD	Exposure pathway retained for further evaluation in the Preliminary Terrestrial Risk Assessment (PTRA).

Table 2-3:	Conceptual Site Model for Ecological Receptors

| Land Use | Receptor of Concern | Receptor Type | COPCs and Operable Pathways |
| --- | --- | --- | --- |
| Wildlands | Ecological Receptors | Terrestrial Populations/ Communities | COPCs for terrestrial ecological receptors identified in soils at the South Waste Rock Pile AEC. Potentially operable exposure pathways include those associated with direct contact with soil COPCs (including dermal contact and ingestion) and indirect exposure (e.g., through ingestion of contaminated food items). |
|  |  | Terrestrial SAR: Northern red-legged frog, Ermine and Wandering salamander identified as SAR with the potential to be present at the Site | COPCs for terrestrial ecological receptors identified in soils at the South Waste Rock Pile AEC. Potentially operable exposure pathways include those associated with direct contact with soil COPCs (including dermal contact and ingestion) and indirect exposure (e.g., through ingestion of contaminated food items). |

Note:

BOLD	Exposure pathway retained for further evaluation in the Preliminary Terrestrial Risk Assessment.

# Preliminary Terrestrial Risk Assessment

Protection of ecological receptors at the community or population level is important for the continued existence of individual species and the health of ecosystems as a whole. Though the primary focus of the ERA is to assess potential effects at the community or population level, in some circumstances, such as for SAR, protection at the individual level may be required. The scope of the PTRA is limited to the evaluation of potential risks to terrestrial ecological receptors at the South Waste Rock Pile AEC. While the current PTRA does not include an evaluation of the potential for contributions of COPCs from these areas to nearby aquatic receiving environments, including to the Jordan River, COPC loading from these areas should be considered in the decision-making process for how to manage or remediate soils at the South Waste Rock Pile.

Soil COPCs for terrestrial ecological receptors were identified at the South Waste Rock Pile and terrestrial ecological receptors, including vegetation (trees, shrubs, herbs), soil invertebrates, birds, mammals, reptiles and amphibians, as well as SAR, have the potential to be directly and indirectly exposed to the COPCs identified. Potential exposures of the COPCs to terrestrial ecological receptors at the South Waste Rock Pile will be briefly discussed and qualitatively evaluated in Section 3.1, below.

## South Waste Rock Pile

Copper and selenium were identified as COPCs for terrestrial ecological receptors in soils at the South Waste Rock Pile (AEC 2), with copper exceeding the BC CSR UCC in both surface and subsurface soil. As previously described in Section 2.4.2, at-grade receptors are most likely to come into contact with surficial soils, with the exception of deep rooting vegetation which may be exposed to soils up to 3 m bgs. The subsurface soil sample exceedances at the South Waste Rock Pile AEC 2 are very deep, which were identified at a depth > 11 m bgs. No operable exposure pathways exist between  terrestrial ecological receptors and contamination at depths > 3 m bgs, and thus, there are no direct contact risks associated with this deep contamination.

As the goal of the PTRA is to identify COPCs that will drive risk assessment outcomes to inform remedial decision-making, 95% upper confidence limits of the mean (UCLMs) COPC concentrations were calculated using US EPA ProUCL Version 5.1, with the selection of 95% UCLM values determined by sample size, the distribution of data and number of non-detect samples in each data set as per recommendations by Helsel and Gilroy (2012). Appendix B provides the UCLM outputs.

The surface soil 95% UCLM concentration selenium was estimated to be 2.68 µg/g. While the 95% UCLM concentrations exceed the BC CSR WLR soil standards for toxicity to plants and soil invertebrates, the selenium concentration is less than the BC ENV (2023d) Protocol 4 regional background concentration of 4 µg/g. On this basis, no unacceptable risks are predicted for selenium for terrestrial ecological receptors at the community to population level.

The surface soil 95% UCLM concentration for copper was estimated to be 1,695 µg/g. The 95% UCLM concentration for copper exceeds the both the BC CSR WLR standard for toxicity to soil invertebrates and plants, as well as the UCC. Though there are elevated concentrations of COPCs identified in piled waste rock at the South Waste Rock Pile AEC, the bioavailability of copper in the waste rock pile is anticipated to be low as the copper toxicity characteristic leaching procedure (TCLP) results have been below standards (SNC-Lavalin, 2018) and due to the physical characteristics (hardened nature) of waste rock. Copper concentrations in excess of the UCC were also identified outside of the South Waste Rock Pile, such as at SS17-07 which is adjacent to the Jordan River (see Drawing 631848-103 and attached analytical tables). Based on the magnitude of exceedances noted in this AEC, copper is likely to drive risks to terrestrial ecological receptors and has been discussed in further detail, below.

The total area of the South Waste Rock Pile AEC is 4,300 m2, with the area of tailings within this AEC that is slated to be remediated encompassing approximately 500 m2. Corvidae (2021) noted that outside of the area of piled waste rock material, mature riparian forest ecosystem is present and was described as “intact” (Corvidae, 2021). While not conclusive, the biological observations from SNC-Lavalin (2016) and Corvidae (2021) for the South Waste Rock Pile AEC suggest that effects to terrestrial ecological receptors, particularly to the habitat provided by terrestrial vegetation, are not likely to be occurring at the population or community level. Additional observations comparing the vegetation at the South Waste Rock Pile to reference areas, with consideration of species distribution and other vegetation assessment metrics and would provide further support to this conclusion.

Three SAR were identified with the potential to be present in the vicinity (< 1.5 km) of the Site, including the Northern red-legged frog, Wandering salamander and Ermine (see Section 2.2.3). Corvidae (2021) identified suitable habitat for the Northern red-legged frog in the riparian corridor adjacent the South Waste Rock Pile . It is likely that this habitat would also be suitable for the Wandering salamander (BC CDC, 2023), while for the Ermine significant use is unclear as this species inhabits a variety of habitat subtypes, including wooded areas adjacent to wetlands and river systems, as well as meadows and agricultural fields. The home range for the Ermine is > 10,000 m2 (FCSAP, 2012) therefore it is likely this species would not be present for long-periods of time at the South Waste Rock Pile AEC. Home ranges for the Northern red-legged frog have been recorded as 100 m2 to 200 m2 (Haggard, 2000; Schuett-Hames, 2004) and based on Maxcy and Richardson (1999) and COSEWIC (2012, 2014) the home range for the Wandering salamander is also likely small (<100 m2), with many staying within 2 m to 10 m of capture sites.

The Northern red-legged frog is most commonly found in shallow ponds, along streams, in wetlands and moist forest (BC CDC, 2008). They require standing water for breeding, in which eggs are attached to emergent vegetation below surface water. This species is negatively associated with sloped hills and elevation and tend to be more abundant in riparian areas near breeding locations (BC MOE, 2004). The most critical component for terrestrial habitat is likely sufficient cover and, on a larger scale, connectivity and distance between wetlands to maintain metapopulation dynamics (BC MOE, 2004). As mentioned previously, most frogs don’t move any significant distance beyond capture points (< 10 m), however significant movement (> 300 m) has been recorded over short periods, likely due to displacement. Potential exposures  to COPC in the South Waste Rock Pile to the Northern red-legged frog are therefore likely only to be through soil contact and invertebrates (diet) and in flatter areas; the unnamed creeks are unlikely to be breeding habitat based on the absence of stagnant pooled water and emergent vegetation, but this should be confirmed during a future Site visit.

Conversely, the Wandering salamander is completely terrestrial throughout its life cycle, lacking an aquatic larval stage (COSEWIC, 2014). From egg to adult, the species requires moist microhabitats, and is almost always found inside cavities or cracks within decaying logs or under loose bark covering logs or stumps, and rarely in contact with the ground (COSEWIC, 2014). As noted in the BC CDC iMap search, extensive tracts of suitable habitat exist in nearby forested lands and along Juan de Fuca Provincial Park, therefore the species is likely to be present in the region. The Wandering salamander is thought to be tolerant of some habitat disturbance because it has been found along the edges of forests and within clearings, burned-over areas, and transmission corridors (COSEWIC, 2014). As noted, this species has a high site fidelity and low rates of active dispersal and movement. Potential exposures to the Wandering salamander are therefore likely only to be through invertebrates (diet) as contact with soil is expected to be minimal, and direct contact with surface water is also unlikely.

While concentrations of copper in surface soil measured in this South Waste Rock Pile AEC are likely to drive potential risks to amphibian SAR if present, the bioavailability of copper in AEC surface soil is not currently well understood and based on experience at other mine sites, has the potential to be relatively low. Additionally, the areas of suitable habitat for these amphibians in the AEC described by Corvidae (2021) relative to the areas being remediated within the AEC is also unclear. As a result, it is recommended that a survey of the riparian corridor be conducted to determine if the amphibian SAR are present, and that detailed mapping of habitat considered to be potentially suitable for SAR amphibians be performed. If these species are directly observed or are determined to have a high likelihood of being present based on the suitability of habitat within the contaminated area, additional surface soil sampling targeting areas of the AEC considered suitable habitat for amphibians is recommended. Co-located samples of invertebrates and plant tissue could also be collected from these locations, to enable an evaluation of the bioavailability of copper in AEC surface soil and to allow for direct inputs of invertebrate tissue concentrations into amphibian exposure estimates. This additional information could be used to support the completion of a DERA for amphibian SAR with the potential to be present in this AEC, if required.

# Conclusions

The objective of the current PTRA was to evaluate potential exposures and risks to human and terrestrial ecological receptors exposed to soil in the South Waste Rock Pile, to support remedial and risk management decision-making for this AEC. The results of the risk evaluation for human health and terrestrial ecological receptors are summarized below.

## Human Health Risks

The Site is accessible via an unpaved and gated forest access road. While there is no formal day use area, signs of recreational use have been noted during Site visits. It was assumed that the Site could be used for traditional or recreational purposes. No COPCs for human health were identified in surface soil at the South Waste Rock Pile AEC, therefore no unacceptable risks to human health resulting from direct exposures to soil in this area are anticipated. Given the lack of COPCs in surface soil, the potential for significant exposures to soil COPCs via indirect exposure, including via consumption of country or traditional foods, is considered to be negligible.

## Terrestrial Ecological Risks

The Site provides significant terrestrial habitat for a diverse array of species, potentially including SAR. COPCs for terrestrial ecological receptors were identified in surface soil (copper and selenium) and subsurface soil (copper) at the South Waste Rock Pile, with copper identified as the COPC likely to drive risks.

An assessment of the terrestrial habitat at the South Waste Rock Pile was completed by Corvidae (2021). While elevated concentrations of copper have been identified in soil in this area, observations from SNC-Lavalin (2016) and Corvidae (2021) suggest that mature riparian forest ecosystem is present and was described as “intact” (Corvidae, 2021). While not conclusive, the biological observations for the South Waste Rock Pile AEC suggest that effects to terrestrial ecological receptors are not likely to be occurring at the population or community level. Further assessment of the area relative to reference areas could be conducted to confirm this finding.

The habitat assessment also identified potentially suitable habitat in the riparian corridor that bisects the North and South Waste Rock Piles for the Northern red-legged frog, an amphibian SAR documented within 1.4 km of the Site. Additionally, this habitat may also be suitable for the Wandering salamander, another amphibian SAR with documented occurrences in the area surrounding the Site. The presence of these amphibian SAR at the South Waste Rock Pile AEC is unknown and requires confirmation. The areas of suitable habitat for these amphibians in the AEC described by Corvidae (2021) relative to the areas being remediated within the AEC is also unclear. As a result, it is recommended that a survey of the riparian corridor be conducted to determine if the amphibian SAR are present, and that detailed mapping of habitat considered to be potentially suitable for SAR amphibians be performed. If these species are directly observed or are determined to have a high likelihood of being present based on the suitability of habitat within the contaminated area, additional assessment, followed by the completion of a DERA, is recommended.

Given the relatively small size of the areas of the Site evaluated herein and the vast productive habitat that surrounds these areas, consideration should be given to the disturbance to ecological receptors that would be required to complete physical remediation at the relatively inaccessible areas of the Site. Additionally, while the current PTRA does not include an evaluation of the potential for contributions of COPCs from these areas to nearby aquatic receiving environments, including the Jordan River, COPC loading from the South Waste Rock Pile should be considered in the decision-making process for contaminated soil management.

# References

BC Conservation Data Centre (BC CDC). 2008. Species Summary: Rana aurora. B.C. Ministry of Environment. Available: https://a100.gov.bc.ca/pub/eswp/ (accessed Dec 4, 2023).

BC Ministry of Environment (MOE). 2004 Accounts and Measures for Managing Identified Wildlife, Identified Wildlife Management Strategy. Version 2004. RED-LEGGED FROG Rana aurora aurora.

BC Ministry of Environment and Climate Change Strategy (BC ENV). 2023a. Protocol 11: Upper Cap Concentrations for Substances Listed in the Contaminated Sites Regulation. Version 5.0. March 20, 2023.

BC ENV. 2023b. Protocol 12: Site Risk Classification, Reclassification and Reporting. Version 6. March 20, 2023.

BC ENV. 2023c. Protocol 1: Detailed Risk Assessment. Version 4.0. March 20, 2023.

BC ENV. 2023d. Protocol 4. Establishing Local Background Concentrations in Soil. Version 14. March 20, 2023.

BC ENV. 2023e. Protocol 20: Detailed Ecological Risk Assessment Requirements. Version 3. February 1, 2023.

Canadian Council of Ministers of the Environment (CCME). 2020. Ecological Risk Assessment Guidance.

COSEWIC. 2012. COSEWIC Assessment and Status Report on the Northern Dusky Salamander Desmognathus fuscus in Canada. Committee on the Status of Endangered Wildlife in Canada.

COSEWIC. 2014. COSEWIC Assessment and Status Report on the Wandering Salamander Aneides vagrans in Canada. Committee on the Status of Endangered Wildlife in Canada.

Corvidae Environmental Consulting Inc. (Corvidae). 2021. Vegetation and Wildlife Surveys and Impact Assessment. For SNC-Lavalin Sunro Mine Reclamation. September 2021.

Contaminated Sites Approved Professionals of British Columbia (CSAP). 2013. CSAP Technical Guidance for Soil Sampling Depth to Characterize Ecological Exposure, BC MoE Policy Decision Summary Issue 11. Prepared by Azimuth Consulting Group. July 2013.

Environment and Climate Change Canada (ECCC). 2019. Federal Contaminated Sites Action Plan, Ecological Risk Assessment Guidance – Module 6: Ecological Risk Assessment for Amphibians on Federal Contaminated Sites. Version 1. December 4, 2019.

Federal Contaminated Sites Action Plan (FCSAP). 2012. Ecological Risk Assessment Guidance. Module 3: Standardization of Wildlife Receptor Characteristics. March 2012.

Haggard, JA. 2000. A radio telemetric study of the movement patterns of adult northern red-legged frogs (Rana aurora aurora) at Freshwater Lagoon, Humboldt County, California. Thesis.

Health Canada. 2010. Part V: Guidance on Human Health Detailed Quantitative Risk Assessment for Chemical (DQRACHEM). Federal Contaminated Site Risk Assessment in Canada.

Health Canada, 2021. Guidance on Human Health Preliminary Quantitative Risk Assessment (PQRA). Federal Contaminated Site Risk Assessment in Canada. Version 3.0.

Helsel DR and EJ Gilroy. 2012. The Unofficial Users Guide to ProUCL4.0. Kindle Ebooks. Out of Print.

Maxcy KA and Richardson J. 1999. Abundance and Movements of Terrestrial Salamanders in Second-Growth Forests of Southwestern British Columbia. In Proceedings of a Conference on the Biology and Management of Species and Habitats at Risk. Kamloops, BC. 1, 295-301.

Meidinger, D. and J. Pojar. 1991. Ecosystems of British Columbia. BC Ministry of Forests. February 1991. Chapter 6: Coastal Western Hemlock Zone.

Schuett-Hames, J. 2004. Northern Red-legged Frog (Rana aurora aurora) Terrestrial Habitat use in the Puget Lowlands of Washington. Thesis.

SNC-Lavalin Inc. (SNC-Lavalin). 2016. Supplemental Stage 1 and Limited Stage 2 Preliminary Site Investigation and Preliminary Risk Opinion. Sunro Mine, Jordan River, BC. June 24, 2016. Prepared for Teck Metals Ltd.

SNC-Lavalin. 2017. Limited Detailed Site Investigation. Sunro Mine, Jordan River, BC. July 10, 2017. Prepared for Teck Metals Ltd.

SNC-Lavalin. 2018. Supplemental Detailed Site Investigation. Sunro Mine, Jordan River, BC. July 27, 2018. Prepared for Teck Metals Ltd.

SNC-Lavalin. 2023. 2023 to 2024 Water Sampling and Risk Opinion Cost Estimate at the Sunro Mine, Jordan River, BC. June 1, 2023. Prepared for Teck Metals Ltd.

Tables

- Soil Log

- Summary of Analytical Results for Metals in Soil

drawings

- 631848-101 – Site Location Plan

- 631848-102 – Areas of Environmental Concern

- 631848-103 – Soil Sample Location Plan

Appendix A

Habitat Assessment and Species Search Results

Appendix B

ProUCL Output

| AtkinsRéalis 1300 - 3777 Kingsway Burnaby, BC V5H 3Z7 Canada 604.662.3555 atkinsrealis.com |
| --- |
| © AtkinsRéalis except where stated otherwise |  |

Formerly known as SNC-Lavalin Inc.

Formerly known as SNC-Lavalin Inc.

Contaminated Sites Regulation (CSR), B.C. Reg. 375/96, includes amendments up to B.C. Reg. 133/2022, March 1, 2023.

Contaminants of concern are defined as substance that is present in media at a site at levels that exceed generic numerical standards prescribed for that media.

Contaminants of potential concern are defined as a substance that is retained for evaluation in a risk assessment.

Andrew Wan, MET, RPBio (#1936).

Jessica Harvey, MSc, RPBio (#2556).